Emergency Procedures

Evacuation - In emergency situations, one of the hardest decisions a driver may face is whether to evacuate a transit vehicle or not.  There are risks involved in an evacuation because of the potential to place a passenger in harm’s way.   Evacuation is recommended at any time when the risks of staying on board the vehicle are greater than the risks involved having the passengers off the vehicle.  If there is evidence of smoke or fire, evacuate the passengers first, then investigate the cause.  If smoke or fire is coming from the engine, never open the hood.

•  Communicate calmly with passengers that evacuation is necessary, indicating which exits they are to use and where they are to gather after leaving the vehicle.

•  If any disabled, frail elderly, children or wheelchair passengers are on board, ask for evacuation assistance from ambulatory passengers.

•  Evacuate all non-assisting ambulatory passengers first instructing one to take the fire extinguisher.

•  When evacuating non-ambulatory passengers:

- Use a web cutter to cut through all securement straps.

- Use the lift, or use the lift, lowered halfway.  Manually operate the lift, if necessary and possible.

- carry non-ambulatory passengers out.

- If the lift is not working, drag or carry non-ambulatory passengers through the door.

- If normal exits are blocked, drag or carry non-ambulatory passengers through emergency exit windows, preferably with assistance from outside and inside the vehicle.

• 	Once all passengers are clear of the vehicle, calmly guide passengers to a safe area, assess their condition and make sure that dispatch and emergency personnel have been contacted.

Note:  Emergency roof hatches and windows are used only in the event that

doors are blocked or the vehicle has rolled on its side.

Communication Procedures in Emergency Situations - The driver must understand how to communicate key facts about emergency situations with the dispatcher or other key response personnel in the transit organization. The driver must assist police, fire, or emergency medical people who respond to a transit vehicle emergency. Part of this communication revolves around liability issues, such as training drivers not to volunteer information about what did – or didn’t – cause an emergency situation to occur. A driver must inform emergency response personnel about vehicle safety features, such as the location and use of emergency exits or the location of emergency shut-offs.

Do not assign blame nor take responsibility for the accident/incident.

Avoid talking to the media, but instead refer the media to senior management.

What And How To Communicate To Passengers - The driver must remain calm while communicating the key facts about the emergency at hand; the driver must communicate his or her intentions about evacuation and other measures clearly and calmly with the passengers. Drivers need to communicate in a manner appropriate to the passengers who are being transported.  Drivers may be helping evacuate persons who are easily confused, have cognitive impairments, or who have sensory disabilities that prevent them from seeing or hearing life-threatening information.

Choosing The Best Routes - Once passengers are evacuated, a driver has the lead responsibility to ensure they gather at a safe place.  The driver should guide all passengers completely out of harm’s way, out of traffic to a protected area, if possible, at a minimum of 100 feet from the vehicle.  If the driver cannot lead this function, they need to designate a competent passenger to guide passengers away from the emergency.

Vehicle Fires

- If smoke or fire is present, EVACUATE the vehicle immediately.

- Contact Dispatch

- DO NOT open up the hood or engine compartment if there are signs of fire inside.

- Shut off all electrical power.

- Only try to extinguish a fire if you are sure of what to do and only if it is safe to do so.  The

fire extinguisher is to be used primarily to create a way off  the vehicle for evacuation purposes.

- Only after the vehicle has been evacuated should the extinguisher be used to put out a fire.

- When using the extinguisher, stay upwind and as far away from the fire as possible

- Aim the extinguisher at the source or the base of the fire, not at the flames.

Use of Fire Extinguisher - Drivers are not required to fight a fire. If there is the slightest doubt about your control of the situation, DO NOT FIGHT THE FIRE.

- Use a mental checklist to make a Fight-or-Flight Decision. Attempt to use an extinguisher only if ALL of the following apply:

- Dispatch and emergency personnel have been contacted.

- The vehicle is evacuated.

- The fire is small, contained and not spreading beyond its starting point.

- The exit is clear, there is no imminent peril and you can fight the fire with your back to the exit.

- You can stay upwind or low and avoid smoke.  Smoke inhalation must be avoided since only a small amount of toxic smoke can render the driver unconscious.

- The proper extinguisher is immediately at hand.

- You know how to use the extinguisher.

IF ANY OF THESE CONDITIONS HAVE NOT BEEN MET, DON'T FIGHT THE FIRE YOURSELF. CALL FOR HELP AND LEAVE THE AREA.

2.   Whenever possible, use the "Buddy System" to have someone back you up when using a fire  extinguisher. If you have any doubt about your personal safety, or if you can not extinguish a fire, leave    immediately and close off the area if possible.

3.   Pull the pin on the fire extinguisher.  Stand at least several feet and upwind from the fire, depress the handle and sweep back and forth towards the base of the fire. Note: Remember: you usually can't expect more than 10 full seconds of extinguishing power on a typical unit.